| Author(s) | BIM for Housing Associations |
| --- | --- |
| Document Name | BIM4HAs Asset Information Requirements |
| Document Status | For public Issue |
| Description | Asset Information Requirements (AIRs) describe the high level information you will need to maintain your built asset throughout its lifecycle and why you’ll need them. This document is a template – use it as a starting point to diagnose and define your own asset information needs to manage your own organisation’s assets. Your organisation should only have one set of AIRs per asset or per asset type. Update it periodically but make sure you transfer this update of requirements across your organisation. |
| Disclaimer | The details in this document are for general information purposes only. Nothing in these documents constitutes legal advice or gives rise to a solicitor/[Insert Client Name] relationship. Specialist legal advice should be taken in relation to specific circumstances. |
| Document Version | 2.0 |
| Date Issued | 10/08/2022 |
| Information | http://housing.org.uk/BIM4HAs |
| Contact | BIM4HAs@ukbimalliance.org |

VERSION HISTORY

| Date | Description | Version |
| --- | --- | --- |
| 21/06/2021 | First Public Issue | 1.0 |
| 26/06/2022 | Partial Update to begin to align with UK BIM Framework |  |
| 10/08/2022 | Second Public Issue | 2.0 |

# [INSERT NAME OF PROJECT]

# [INSERT NUMBER OF PROJECT]

# ASSET INFORMATION REQUIREMENTS (AIR)

# Document Control

| Revision | Date | Revision Details | Revision By |
| --- | --- | --- | --- |
| 1.0 | MM.YYYY | Issued for Use |  |

# AIR – Document Overview

This document is a guideline for [INSERT COMPANY NAME] to detail the Asset Information Requirements (AIR) in a consistent manner across all assets that come under the organisation.

It is recommended that in forming the AIR, that the relevant stakeholders undertake a workshop at project inception stage, to detail the aspirations and information requirements of the Asset in simple non-technical terms. The AIR is for asset management/operation. A by-product of the AIR is that it will be used to support the development of the EIR.

The outputs from this workshop will form the basis for the AIR and has allowed for the creation of the Exchange Information Requirements (EIR).

# Document Ownership

[INSERT COMPANY NAME] has overall ownership of this document and the control of all information ensure the outputs are fully delivered. This will be the information manager appointed by [INSERT COMPANY NAME].

# Document Authority

All Appropriate representatives shall, with the authority of their appointed organisations, agree to accept this document as the AIR’s. This acceptance will confirms that the relevant personnel in the supply chain have read and understood its requirements and agree to abide by the requirements herein.

The document provides structure and specifications in accordance with the UK BIM Framework

This document is required for the development of the Exchange Information Requirements (EIR).

Contents

AIR – Document Overview	1

Document Ownership	1

Document Authority	1

Document Purpose	5

Intellectual property rights	6

Classification	6

Government Soft Landings	6

General Information Requirements	7

Project Handover Control Sheet	9

Health and Safety Files	9

Operations and Maintenance (O&M) Manuals	10

Building Emergency Folder	10

2D Drawings	11

Masterplan Drawings	12

Building Information Model	13

Construction Operation Building information exchange (COBie)	13

Fire Safety Folder	14

Building Safety Case	14

Built Estate Record Information	16

Operational Estate Information	20

Lead appointing Party Requirements	27

Appendix A - Operational & Maintenance File Template	33

Appendix B – Project Information/ Health and Safety/ Building Manual Handover Checklist	35

1.1	Appointment	35

1.2	Statutory Approvals	35

1.3	Health & Safety File	36

1.4	As-built Drawings / Design Information (Important: Product drawings must be held with relevant O&M)	36

1.5	Existing Information	37

1.6	Operation and Maintenance Manuals	42

Water Management Policy	54

1.2 Policy Statement		54

1.3 Policy Implementation		54

Appendix C – Project Information	56

Appendix D- Key Schedule Sheet	65

Communal Key Schedule (all blocks should have 2 sets of landlord supply keys)	65

Landlords/Communal Supply Details (please add lines if more than one meter)		65

Appendix E – Property Terrier Template	68

Appendix F – Building Lifecycle Data Return	69

Appendix G - Asset Hierarchy	74

Appendix H - COBie Requirements	75

Appendix G - CAD Standards Checklist	81

Appendix H - CAD Standards - Declaration of Compliance	81

Appendix I - Asset Information Requirements – Declaration of Requirements	83

# Document Purpose

This document sets out the Asset Information Requirements (AIR) of [INSERT COMPANY NAME], in line with the standards represented within the UK BIM Framework https://www.ukbimframework.org/, specifically ISO 19650-3.

ISO19650 defines AIR as 'information requirements in relation to the operation of an asset'.  The AIR sets out high level information requirements and an information requirement is defined as the 'specification for what, when how and for whom information is to be produced’. This AIR guidance document sets out the requirements for information to meet the needs of [INSERT COMPANY NAME]. The AIR is an internal facing resource and once established it will be communicated through the EIR (external facing resources) to enable the creation of an Asset Information Model (AIM). The relationship between the different information requirements and the information models is as follows:

Organisational Information Requirements (OIR)

Information requirements at an organisational level.

Asset Information Requirements (AIR)

AIR focuses on information that the Appointing Party requires at key decision points during the operation and maintenance of an asset

assets.

Exchange Information Requirements (EIR)

Sets out the information to be delivered.

Asset Information Model (AIM)

The single source of information related to an asset at a level required to support an organisations asset management system.

Project Information Requirements (PIR)

PIR focuses on information that the Appointing Party requires at key decision points during a design and construction project.

Project Information Model (PIM)

The single source of information related to an asset developed during the design and construction phase of a project.

This can also be seen in figure 2 of ISO 19650-1 (below).

# General Information Requirements

Core Information Requirements

The following information can be split into core asset information requirements under the different types of information

Managerial information

- Photograph

- Type of Asset

- Location

- Floor area

- Warranties and Guarantees

- Maintenance and Inspection Schedules

- Standards, Processes and Procedures

- End of life processes

- Emergency Plans

Commercial Information

- Description

- Function

- Details of supplier

- Performance targets or standards

Technical

- Engineering data

- Design parameters

- Interdependencies

- Commissioning dates and data

- Operational data

- Finishes

- Services requirements

Linking AIR, OIR and AIM

ISO 19650-3 shows the relationship and progression of information requirements as set out in figure 2 of ISO 19650-3

# ISO 19650 Equivalency Table with general Housing Association Terminology

Below sets out a typical set of definitions which can be mapped to the ISO 19650 Terminology. This information will make it easier to understand the terminology being used and for further information the ISO 19650-0 Transition guide should be read to further understand additional terminology that is used within the standards of the UK BIM Framework.

| ISO 19650 Terminology | Housing Association Terminology |
| --- | --- |
| Risk Register | Risk Register |
| Delivery | Delivery Phase |
| Appointment | Contract |
| Appointing Party | Employer/Client |
| Geometrical/Non geometrical | Graphical and Non-Graphical |
| Appointed Party | Design Team and Sub Contractors (any organisation generating information) |
| Lead Appointed Party | Lead Designer/Main Contractor (any organisation appointed by the Employer/Client) |

# Handover Control Sheet

General Requirements (what)

[INSERT COMPANY NAME] will be provided with a handover control schedule on completion of a trigger event or project. This will clearly identify what information is provided at that handover point and will be included in the Operations and Maintenance Manuals (O&M).  This should be done for completion of project and operation and maintenance activities.

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required (why)

This information will be used as a checklist to ensure all information requirements (EIR) are met (including requirements for COBie).

Format (how)

Microsoft Excel format.

The template for the handover control sheet is in [   ] HA to complete and append their template to the document and will be provided for each project or trigger event.

# Health and Safety (H&S) Files

General Requirements

The H&S File shall be provided for all relevant projects and trigger events by the principal contractor (a lead appointed party).   It shall be scaled to suit the size and complexity of the project or trigger event as well as the residual risks. The H&S file shall contain the information that will enable [INSERT COMPANY NAME] to safely operate and maintain its facilities and meet statutory requirements now and the future.

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To enable [INSERT COMPANY NAME] to fulfil its CDM 2015 duties.

Format

PDF format that shall not exceed 50mb.

The H&S files shall include the items listed within Appendix B section 1.3 this document. Where an item is not applicable this shall be stated by [INSERT COMPANY NAME] .

# Operations and Maintenance (O&M) Manuals

General Requirements

The O&M Manuals shall be provided for all relevant projects and trigger events and shall be scaled to suit the size and complexity of the project or trigger event as well as the residual risks. The file shall contain information that will enable [INSERT COMPANY NAME] to safely operate and maintain its facilities.

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To support the operation and maintenance of the asset.

Format

Bookmarked .PDF format. Each file shall not exceed 50Mb in size. If the file does exceed this size, it shall be split into volumes (e.g. volume 1, volume 2 and so on). With bookmarks to identify the content.

The O&M Manuals shall include the sections and information listed (specify the appendix within the Employer Requirements which will have these).  Any section of the O&M File which is not relevant to a particular project or trigger event shall be retained but shall be noted as ‘Not Applicable’ and left blank.

# Building Safety Emergency Folder

General Requirements

A building emergency folder shall be provided and kept up to date for each asset.  It shall be displayed in the foyer/main entrance of an asset and provides information for emergency services on the layout of the buildings, any high-risk items, location of gas cylinders, radioactive materials, hazardous chemicals, and any evacuation routes that are applicable to the building. For each relevant project or trigger event the building safety emergency folder must be populated with new or updated fire plan drawings as well as fire alarm cause and effect drawings.

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

The information provided is required to form part of the Building Safety Case for this asset.

Format

PDF format shall be provided, clearly marking hazardous and high risk areas, as well as plans with evacuation areas and routes clearly marked.

Copies of the drawings listed in Appendix TBC by the Appointing Party shall be provided to [INSERT APPOINTING PARTY NAME] as part of the Building Safety Emergency Folder.

# 2D Drawings

General Requirements

- Drawings shall be provided at each Information Exchange.

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

Drawings shall be required for approval of works, to support Building Safety Gateways, making informed decisions, for tender and construction/installation and as a record of work done. It is essential information is updated throughout the asset lifecycle.

Format

Drawings shall be provided electronically in .dwg format and PDF format.

# Geometrical 3D Model

General Requirements

A federated geometrical model at agreed Information Exchange(s).

Stakeholder/information recipient (who)

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To support the operation and maintenance of the asset.

Format

Geometrical model files shall be provided in native file format and IFC 2x3 coordination view.

# Construction Operation Building information exchange (COBie)

General Requirements

The scope of standard structured asset and project data shall be provided as indicated in the COBie table below.  It shall be provided in accordance with BS 1192-4.  Below sets out the fields which are required, expected and any additional attributed fields which will be required.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To support the operation and maintenance of the asset.

Format

The data shall be provided in Microsoft Excel .xlsx format. Certificates and all other supporting information shall all be provided in .PDF format and referenced in the document tab.

The data shall be exported from the relevant geometrical models, and it shall be supplemented as required.

# Fire Safety Folder

General Requirements

The project team shall provide a work in progress Fire Safety folder which shall contain information relating to all fire safety aspects within the asset. This should begin with the Fire Statement at Gateway 1 and continue to develop throughout Gateway 2 and Gateway 3, with a fully completed Fire Safety Folder which would include Fire Strategy, Risk assessment and Evacuation Strategy All associated documentation required within the document shall be provided as an electronic file format 2 weeks prior to handover for validation checks to ensure the information provided is correct and content is in the correct format for operational use. The Fire Safety Folder prior to handover should be kept on the common data environment and developed as work in progress.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

This information will be utilised to keep fire safety information up to date throughout the asset lifecycle. It is important that this is kept up to date to ensure successful advancement of the Building Safety Gateways and to maintain the Golden Thread.

Format

The fire Safety folder shall be provided in.Docx and .PDF formats. 2D Details, Drawings and all associated documentation shall be provided in .PDF format..

# Building Safety Case and Safety Case Report

General requirements

Required for higher risk buildings. It should refer to the totality of the building safety information and include all supporting evidence identifying how fire and structural risks are being managed as well as other building safety responsibilities required under applicable statutory obligations placed on the building owner.  At Gateway 2 the beginning of the safety case will be developed, and it will be updated at Gateway 3. It shall include annotations and sketches as well as having embedded links to an online digital platform providing the necessary safety case information.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME] and the Building Safety Regulator plus the Principal Accountable Person

Purpose for which information is required

For higher risk buildings this will be a requirement in order enable occupation and prove the building is safe for residents. It sets the benchmark for how the Golden Thread of Information will need to be delivered for the remainder of the asset lifecycle.

To demonstrate to the Building Safety Regulator the safety and managed risks associated with an asset to obtain a Building Assurance Certificate prior to practical completion.

Format

The Building safety case report shall be provided in .Docx and .PDF formats.

# Record Information

## Space Information

General Requirements

An up to date record of all spaces including space name, space function and spatial dimensions.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

So that space management records can be maintained and to support lifecycle costing and so that rents and ground rents associated with these spaces can be managed. Format

.xlsx format

## Asbestos Register

General Requirements

An up to date record of all work undertaken to remove, modify or encapsulate asbestos-based materials in accordance with [INSERT COMPANY NAME] Asbestos Management Plan.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

The Lead appointed party will be responsible for providing this information which should be referenced in Health and Safety files, risk register and included within the COBie data schema under the documents section.

Purpose for which information is required

To fulfil our statutory requirements and to provide a record of the asbestos within the estate.

Format

.xlsx format.

## Property Terrier

General Requirements

Information, in the form of a set of schedules to update the Property Terrier.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

This information will be utilised to keep a record of the estate, spaces within an estate, rents and ground rents associated with these spaces. It will also be used to inform future refurbishments, any major works and lifecycle costing activities.

Format

.xlsx format structured according to the Property Terrier template

## Planning and Other Consents

General Requirements

Record of statutory approvals (planning and listed building consents, building regulation approvals, English Heritage consent etc.).

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To provide a record of discharge of statutory obligations.

Format

.PDF format and hard copy

## Testing and commissioning certificates

General Requirements

All testing certificates relating to materials, components structure, plant and equipment [insert additional requirements as applicable].

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

This information is required for the operation and maintenance asset.

Format

Each individual certificate shall be provided in .PDF format.  Each certificate shall be referenced in FIREie (for relevant components) and COBie (where a certificate relates to a maintainable component).Testing data should be provided through COBie where available to be read in conjunction with the certification.

## Warranties

General Requirements

All warranties relating to materials, components, structure, plant and equipment [insert additional requirements as applicable].

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

This information is required for the operation and maintenance of the asset.

Format

Each warranty shall be provided in .PDF format.  Each warranty shall be referenced in FIREie (for relevant components) and COBie (where a warranty relates to a maintainable component).

## All Wayfinding: Signage Asset Register

General Requirements

Details of signage, removed signage, and alterations to signage for the asset.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To populate the Appointing Party’s Wayfinding Signage Asset Register and to support the operation and maintenance of the asset.

Format

.xlsx format.

# Operational Information

Background

[INSERT COMPANY NAME] aims to deliver a maintenance regime that ensures, buildings are safe, compliant and fit for purpose. In support of this aim we have developed an annual planned preventative maintenance (PPM) programme which ensures that routine servicing, statutory inspection, and testing is carried out on maintained assets in a timely prescribed manner.

At the core of the PPM programme is the ‘asset maintenance register’, which records the assets and systems that are subject to cyclical maintenance and inspection. Routine maintenance ‘tasks’ are then allocated to the ‘assets’ in order to generate the PPM programme. The full and timely provisions of the asset register is a key element of a successful project handover.

A maintainable component is one that requires periodic maintenance, inspections and testing by or on behalf of [INSERT COMPANY NAME].

A maintainable ‘system’ is defined as a collection of maintainable components that make up a technical system requiring periodic maintenance, inspection or testing by or on behalf of [INSERT COMPANY NAME].

## Maintained Assets and Planned Preventative Maintenance

General Requirements

COBie deliverable containing details of the maintenance and inspection tasks for maintainable components plus a record of information no longer needed in the asset management system(s)

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To update the asset management system(s) for operation and maintenance.

Format

For structured data to be received: COBie .xlsx spreadsheet.  Supporting documents (for example method statements, specification information and maintenance instructions) shall be provided as .pdfs

For data that needs to be removed: .xlsx based on asset register record, provided as reference information by the Appointing Party [INSERT COMPANY NAME]

## Security Systems (Intruders Alarms, CCTV, Access Control)

General Requirements

Information identifying additions, alterations, and omissions to/from existing security components and systems plus maintenance information.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To support the management of the security of the asset and all those using the asset.

Format

Security System Installation data shall be provided in COBie.  Records of the installations (commissioning records etc.) shall be provided in pdf format with data presented within the job tab within COBie and the document within the document tab for reference purposes.

## Building Life Cycle Data

General Requirements

A schedule of the life expectancy for major items of the asset that are not maintainable components (and have therefore not been captured through the COBie schema)

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To enable strategic decision making about PPM interventions and the impact on replacement regime and to inform future selection of materials and components.

Format

.xlsx format

## Water Quality and Legionella Control

General Requirements

A legionella risk assessment

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To fulfil statutory obligations associated with legionella.

Format

.PDF

## Cleaning Information/Strategy

General Requirements

A proposed cleaning strategy including details of the cleaning regime

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To enable cleaning activities to be incorporated into the asset management plan and cleaning contracts to be considered.

Format

.PDF format

## Key Schedule

General Requirements

Schedule of all keys for the asset, differentiating the number of master and sub master keys provided. Suiting details and key reference numbers shall also be included with the schedule.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To provide a record of keys provided

Format

.xlsx format as part of the O&M manuals.

## Metering Information

General Requirements

Full details of all utility meters (fiscal and sub-meters) that have been added, altered, or removed as part of the work. Information to include the meter reference (MPR, MPAN, MSN), details of the location of the meter plus meter reading - all recorded in COBie.  A photograph of the meter and access details, including any access restrictions shall also be provided along with schematic drawings (pdf) showing the configuration of parent/child meters.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To provide a record of all meters and meter readings

Format

.xlsx format as part of the O&M manuals.

## Building Management System (BMS)

General Requirements

Full details of any additions, alterations of omissions to the BMS at handover. The information shall include a full description of the BMS and its operation, new or amended control panel schematic drawings, a list of critical BMS alarms configured at handover and a list of panel points.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

Effective maintenance of the asset.

Format

.pdf format as part of the O&M Manuals.

## Energy Performance Data

General Requirements

Energy Performance Certificate (EPC) where relevant plus details of any Low and Zero Carbon technologies, applicable subsidies to be claimed (e.g. feed in tariffs), information about generating equipment that has been installed including design output and details of the modelled energy performance of the asset and operating assumptions.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

This information is required for the performance measurement and lifecycle data of an asset.  The data will be used to measure against targets set as part of Government Soft Landings.

Format

.pdf format and referenced in the document section of COBie and included in the relevant section of the O&M manuals. Energy performance data shall be captured in the Impact section of COBie.

## Insurance Cover and Fixed Asset Valuation

General Requirements

Information to enable appropriate insurance cover to be obtained. Information to include relevant dates (for example, the date of practical completion for a project) and estimates for [insert scope to estimate to be provided].

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To appropriately insure the asset/site.

Format

.pdf included as part of the O&M Manuals.  Estimate data to be included in the Attribute section of COBie.

## Statutory Inspection/Written Scheme Insurance Inspection

General Requirements

Written Scheme of Examination for all ‘pressure systems’ as covered by the HSE that requires statutory inspection/written scheme of insurance inspection/written scheme of insurance inspection.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To comply with the Pressure Systems Safety Regulations 2000 and insurance requirements

Format

.docx included as part of the O&M manuals. The written scheme of examination shall include the following information:

- Fabric information

- Examinations to be carried out

- Frequency of examinations

- Certification date

## Operational Training and Instruction

General Requirements

Written instructions about the operation, inspection and maintenance of the asset and its site. Separate written instructions are required for maintenance and [insert company name] staff involved in routine maintenance to include cause and effect guidance.

Stakeholder/information recipient

The Appointing Party [INSERT COMPANY NAME]

Purpose for which information is required

To train both operational users and those involved in any routine maintenance and inspection.

Format

.docx included as part of the O&M manuals.

# Appointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppointing PartyAppendix A – Example Operational & Maintenance File Template

The Operation and Maintenance Files that is currently used with the organisation is to be inserted here.

| Operational & Maintenance File Template |  |
| --- | --- |
| Cover | Project Title/Building Ref/Job Number/Issue Date/Version Control Data |
| Contents | The manual will contain the follow information, in order listed below |
|  | - Project Title |
|  | - Construction and design team’s details (address, telephone, fax, email, etc.) |
|  | - Details of specialist appointed parties or suppliers (address, telephone, fax, email, etc.) |
|  | - Emergency call out details (Normal hours and out of hours) |
|  | - Block Plan of building indicating utility service entry points if applicable |
|  | - Sheet containing an index of the contents of the manual |
| Section 1 | Original Specification and a summary of any changes to the specification. |
| Section 2 | Description of systems – include principles of operation, design criteria, design temperatures and air changes. |
| Section 3 | Planned and Equipment inventory – This should include manufactures address, telephone and email details. This section should include the fixed asset insurance variation. |
| Section 4 | Maintenance & Operational Requirements - This section should include: Details of planned preventative maintenance work with a full description of the preventative maintenance task and the recommended frequency in order to maintain all warranties. This section should also include a description of maintenance agreements applicable during the defect liability period. Schedule of plant that requires periodic statutory inspection. Schedule of estimated life cycle replacement for major items of fabric and plant. Cleaning strategy for all floor and other surfaces that will require routine cleaning and periodic deep cleaning. Details of training and instruction for operational facilities and maintenance staff. Details of metering Details of BMS |
| Section 5 | Manufactures literature (indexed) including electric supply, controls and ancillaries. |
| Section 6 | As built drawings (indexed and formatted as described in section 4-6). |
| 6.1 | Architectural details (including fabrication details, specialist installations, lintel and ironmongery, key schedule and suiting details). |
| 6.2 | Structural details |
| 6.3 | Mechanical and electrical system details |
| 6.4 | Specialist systems (including fire alarm, emergency lighting, access control, intruder alarms and CCTV, lighting controls etc. |
| Section 7 | Commissioning results (indexed) relating to all items of mechanical and electrical plant and systems. This should include test results, pressure test sheets and boiler test certificates, combustion tests etc. |
| Section 8 | Other tests including the results of statutory test and inspections, e.g. electrical test certificates, legionella, test results, pressure test sheets and boilers test certificates etc. |
| Section 9 | Warranties and spares. This section will include details and copies of warranties and spares provided for all elements of the building fabric, mechanical and electrical system and plant, fittings and equipment. |
| Section 10 | Health and Safety and emergency procedures, including residual hazards and specific decommissioning procedures. This section should also include any location specific risk assessments for all plant rooms and roof areas that may require access and also for maintenance procedures. |
| Section 11 | Environmental Information: This section should include designed environmental performance data, details of energy modelling undertaken at a stage, details of low zero carbon technologies installed, full details of any technical energy systems. Copy of EPC Certificate. Details of any environmental impact assessments, waste disposal records, including records for all hazardous waste disposed of by the project. |